Nanometrics Software and Directory Structure
Directory Structure

C:\Atlas\bin

- Atlas Run Files.

C:\CoreEarthworm\run_test\params




- Working directory for Earthworm

C:\nmx\bin

- all executable files 




– add this directory to the computer’s path

C:\nmx\logs

- Log files from Nanometrics Programs.

C:\nmx\manuals
- Nanometrics manuals in PDF.

C:\nmx\releases
- original software files before installation.

C:\nmx\ringbuffer
- all seismic data 




– backup

C:\nmx\user

- all user configurable files – naqs.stn, *.ini – backup

Software

Alert Mailer

Utility program which forwards alert messages from NaqsServer 



and other programs to a list of email recipients.  
Atlas


- Data Analysis Tool
AutoDRM

- Automatic Data Request Manager, Requests seismic data via 



email, and provides an email response.

Apollo


- User Interface software for Taurus based equipment.
CoreEarthworm
- Subset of the USGS Earthworm Location Software
DataServer

- Package required to provide data from ringbuffers into Atlas
DLL


- Dynamic Link Libraries
GenUtil

- Utilities such as watchdog, net time.
Hypoinverse

- A module of Earthworm used by both Earthworm and Atlas for 



locating events.
IA Mailserver

- Mail Server required for AutoDRM
Laser view

- Software for printing waveforms on a laser printer.
NaqsClient

- Client Tools – includes Waveform data viewing software.
NaqsServer

- Data Acquisition Software
NpToNmxp

- Receives NP data and log packets from a Taurus, and translates 



them into Nmxp packets to be read by NaqsServer. 

Playback

- Data manipulation Tools

