SLIP is supported on serial port 1/2 on the Taurus.

To configure SLIP on the Taurus, the following parameters must be set in /etc/serial_2.conf (for Serial 2)

| Parameters | Possible values | Description |
| --- | --- | --- |
| networking | slip none | SLIP or no networking support (PPP will be implemented in the future) |
| speed | any commonly supported rate e.g. 9600, 14400, 19200, 38400, 57600 | Serial port baud rate |
| proto | slip cslip | Choose from SLIP or Compressed SLIP for better bandwidth utilization |
| localip | any valid ip address e.g. 192.168.1.34 | IP address of the local SLIP interface |
| remoteip | any valid ip address e.g. 192.168.1.35 | IP address of the remote SLIP interface |

This allows for basic routing of packets to the remote host.

For more complex routing requirements, consult Nanometrics support.

TODO:

Hardware flow control

Modem support